AUTOBOT

Complete Build, Configuration & Visual SLAM Guide

Raspberry Pi 5 • Ubuntu 24.04 Noble • ROS 2 Jazzy • ESP32-WROOM-32 • Mecanum chassis

Latest conceptual wiring diagram including encoder-equipped TT motors. Verify actual board pads and pins before applying power.

Document note: this guide consolidates recorded project steps and commands. It distinguishes reported working configurations from items still requiring verification on the current hardware.

# 1. Final architecture and status

| Subsystem | Role / notes |
| --- | --- |
| Raspberry Pi 5, Ubuntu 24.04, ROS 2 Jazzy | High-level computer for sensor drivers, TF, odometry fusion, SLAM, autonomy and RViz. |
| ESP32-WROOM-32 DevKit | Low-level motor controller, Bluetooth command handling and UART command receiver. |
| 2 × OPEN-SMART DRV8833 | Dual H-bridge boards; proposed one board per side, only if board current/thermal ratings support the motors. |
| 4 × TT motors with encoders, mecanum wheels | Motor control from ESP32; encoder A/B feedback routed to Pi 5 for odometry. |
| Intel RealSense D421 | Rigidly chassis-mounted depth/stereo camera, USB to Pi 5; no conventional RGB stream assumed. |
| RPLidar C1 | USB to Pi 5; LaserScan topic expected at /scan. |
| BNO085 | I²C to Pi 5; IMU topic expected at /imu/data. |
| Two 7.4 V LiPo packs and buck converters | Proposed separate motor and electronics supplies; regulated rails and common signal ground required. |

Recorded status: ROS 2 Jazzy, RPLidar C1, BNO085 package, wheel_odometry, robot_bringup and RealSense driver were used in development. The D421 infrared stereo topics were observed. The saved RTAB-Map launch still used a legacy /image_raw RGB input, so stereo visual SLAM must be reconfigured and verified before calling it complete.

# 2. Hardware selection and considerations

## Raspberry Pi 5

Runs ROS 2 sensor drivers, state estimation and mapping. Provide a stable regulated 5 V supply with peak-current headroom and cooling. Simultaneous LiDAR, stereo camera, point clouds and SLAM can load CPU and USB bandwidth.

## ESP32 and DRV8833

ESP32 replaces the Pi Zero WH motor-control role. Confirm exact OPEN-SMART board pad labels and current/thermal limits against TT motor stall current. Startup/stall current can be much higher than free-running current. Verify any sleep/enable pins on the actual breakout.

## TT encoders and mecanum drive

Encoders provide wheel feedback. Confirm encoder voltage before connecting to Pi GPIO; 5 V outputs must not go directly to 3.3 V Pi GPIO. Calibrate ticks/revolution, gearbox ratio, wheel radius, wheel geometry and direction. Mecanum odometry needs all four wheels and roller orientation; differential-drive equations do not model lateral travel.

## D421, RPLidar and BNO085

The D421 is depth/stereo, not conventional RGB. Use rectified left/right infrared streams for stereo visual SLAM. RPLidar C1 connects over USB. BNO085 connects over I²C; verify breakout voltage and SDA/SCL labels. Sensor transforms must match measured mounting positions.

# 3. Install Ubuntu 24.04 Noble on Raspberry Pi 5

Download the official 64-bit Ubuntu 24.04 LTS image for Raspberry Pi from https://ubuntu.com/download/raspberry-pi.

Use Raspberry Pi Imager or a trusted image writer to flash the image to the intended microSD/SSD. Imaging erases the selected drive.

Complete first-boot setup, set hostname/user/password and connect to the home network; Ethernet is useful during setup.

sudo apt update

Refreshes the APT package index.

sudo apt full-upgrade -y

Installs available updates and dependency changes.

sudo reboot

Reboots after kernel/system updates.

lsb_release -a
uname -m
lsb_release -cs
hostname -I

Checks OS release, CPU architecture, codename and IP. Expected codename is noble and architecture aarch64.

## Optional SSH access

sudo apt install -y openssh-server
sudo systemctl enable --now ssh
systemctl status ssh

Enables same-network SSH; connect from another machine with ssh username@<Pi-IP>. Do not expose SSH directly to the public internet.

# 4. Install ROS 2 Jazzy

If Jazzy is already installed, verify first rather than repeating repository setup. Follow the current official Jazzy instructions if key/repository steps change: https://docs.ros.org/en/jazzy/Installation/Ubuntu-Install-Debs.html

sudo apt install -y software-properties-common curl gnupg lsb-release
sudo add-apt-repository universe

Installs repository tools and enables Ubuntu Universe.

sudo curl -sSL https://raw.githubusercontent.com/ros/rosdistro/master/ros.key -o /usr/share/keyrings/ros-archive-keyring.gpg

Installs the signing key used by APT to authenticate ROS packages.

echo "deb [arch=$(dpkg --print-architecture) signed-by=/usr/share/keyrings/ros-archive-keyring.gpg] http://packages.ros.org/ros2/ubuntu $(. /etc/os-release && echo $UBUNTU_CODENAME) main" | sudo tee /etc/apt/sources.list.d/ros2.list > /dev/null

Adds the ROS repository for the installed Ubuntu codename and architecture.

sudo apt update
sudo apt upgrade -y
sudo apt install -y ros-jazzy-desktop ros-dev-tools

Installs Jazzy desktop (including RViz2) and developer tools. Headless systems may use ros-jazzy-ros-base and install visualization separately.

source /opt/ros/jazzy/setup.bash
printenv | grep ROS
ros2 --help

Loads ROS environment variables and checks the CLI.

echo "source /opt/ros/jazzy/setup.bash" >> ~/.bashrc
source ~/.bashrc

Sources Jazzy automatically in new interactive Bash terminals.

sudo apt install -y python3-rosdep python3-colcon-common-extensions python3-vcstool
sudo rosdep init
rosdep update

Installs dependency/build tools. If rosdep init was already run, skip it and run rosdep update.

ros2 doctor --report

Reports ROS environment diagnostics.

# 5. Inspect devices, I²C and permissions

lsusb
ls -l /dev/ttyUSB* /dev/ttyACM* 2>/dev/null
ls -l /dev/i2c* /dev/gpiochip* 2>/dev/null

Lists USB, serial, I²C and GPIO interfaces. Device names can change after reboot.

sudo apt install -y i2c-tools
i2cdetect -y 1

Scans I²C bus 1. If no address appears, check power, SDA/SCL, bus enablement, breakout voltage and the actual bus.

sudo usermod -aG dialout $USER
sudo usermod -aG i2c $USER

Adds the user to serial/I²C groups where available; log out/in for changes. Avoid permanent chmod 777 permissions.

Historical issue: lgpio reported “can not open gpiochip”. Check Pi 5 GPIO library compatibility, device permissions, selected gpiochip and actual pin configuration. Earlier BNO085 I²C scan showed no address; check wiring and power before software debugging.

# 6. ROS workspaces and custom packages

Recorded workspace layout (use paths that actually exist on the current Pi):

~/Desktop/bno085_ws/             # imu_node_pkg
~/Desktop/odom/ros_ws/           # wheel_odometry
~/Desktop/robot_bringup_ws/      # robot_bringup
~/Desktop/slam/                  # sllidar_ros2 source workspace, if built from source

Place package folders inside each workspace src directory. Do not build an empty workspace or duplicate a package installed from apt.

cd ~/Desktop/bno085_ws
rosdep install --from-paths src --ignore-src -r -y
colcon build --symlink-install
source install/setup.bash

cd ~/Desktop/odom/ros_ws
rosdep install --from-paths src --ignore-src -r -y
colcon build --symlink-install
source install/setup.bash

cd ~/Desktop/robot_bringup_ws
rosdep install --from-paths src --ignore-src -r -y
colcon build --symlink-install
source install/setup.bash

cd ~/Desktop/slam
rosdep install --from-paths src --ignore-src -r -y
colcon build --symlink-install
source install/setup.bash

rosdep installs package.xml dependencies; colcon build builds packages; --symlink-install helps development; source makes the overlay discoverable. Build only workspaces that really contain packages.

| Package | Expected role/output |
| --- | --- |
| imu_node_pkg / bno085_node | BNO085 data on /imu/data. |
| wheel_odometry / wheel_odom | Encoder odometry on /odom_raw; verify Pi 5 GPIO and mecanum support. |
| robot_bringup | Sensor launch, static TF and EKF; saved launch includes legacy camera/mapping settings. |
| sllidar_ros2 | RPLidar C1; expected /scan. |
| robot_localization | EKF /odometry/filtered and configured odom → base_link TF. |
| slam_toolbox | 2D LiDAR mapping. |
| rtabmap-ros | Graph mapping and stereo visual SLAM after correct configuration. |

# 7. RPLidar C1

source /opt/ros/jazzy/setup.bash
source ~/Desktop/slam/install/setup.bash
ros2 launch sllidar_ros2 sllidar_c1_launch.py

Starts the recorded C1 launch. If not found, inspect installed package files:

ros2 pkg prefix sllidar_ros2
ros2 pkg executables sllidar_ros2
find "$(ros2 pkg prefix sllidar_ros2)/share/sllidar_ros2" -maxdepth 3 -type f

ros2 topic list | grep scan
ros2 topic info /scan
ros2 topic echo /scan --once
ros2 topic hz /scan

Checks scan topic, publisher/type, one message and frequency. Confirm header.frame_id and matching TF.

Saved example base_link → laser transform: (0.18, 0, 0.12) m with zero rotation. This is not a measured final value; measure the actual sensor position and orientation.

# 8. BNO085 IMU

source /opt/ros/jazzy/setup.bash
source ~/Desktop/bno085_ws/install/setup.bash
ros2 pkg executables imu_node_pkg
ros2 topic info /imu/data
ros2 topic echo /imu/data --once
ros2 topic hz /imu/data

Checks executable and IMU messages. Validate units, frame, axes and covariance before EKF fusion. Ubuntu Noble blocks unmanaged system pip installs; prefer apt, a virtual environment or package-specific dependency instructions.

# 9. Encoders and odometry

Historical pin notes from the earlier Pi 5 implementation: RR GPIO17/27, FR GPIO6/5, RL GPIO16/26 and incomplete FL mapping. Treat this as historical only; verify the current harness and source code.

source /opt/ros/jazzy/setup.bash
source ~/Desktop/odom/ros_ws/install/setup.bash
ros2 topic echo /odom_raw --once
ros2 topic hz /odom_raw

Checks raw odometry. Lift the chassis, rotate each wheel individually, verify counts/signs, then calibrate ticks/revolution, gear ratio, wheel radius and geometry.

Mecanum needs all four wheel measurements and correct roller orientation/sign convention; differential-drive odometry cannot model sideways movement.

Encoder outputs must be safe for 3.3 V Pi GPIO. Add level shifting/buffering if required and protect signals from motor noise.

Encoder signals go to Pi 5 in the updated plan, while ESP32 drives motors. Final GPIO assignments and mecanum code need validation; do not assume the old node is already compatible.

# 10. Bringup, EKF and TF

source /opt/ros/jazzy/setup.bash
source ~/Desktop/bno085_ws/install/setup.bash
source ~/Desktop/odom/ros_ws/install/setup.bash
source ~/Desktop/slam/install/setup.bash
source ~/Desktop/robot_bringup_ws/install/setup.bash
ros2 launch robot_bringup bringup.launch.py

Sources required overlays and starts saved bringup. Source only workspaces that exist. If a package is missing, use ros2 pkg prefix <package> and rebuild/source the correct workspace.

Recorded EKF: 50 Hz, 2D mode, odom0=/odom_raw, imu0=/imu/data, world_frame=odom, odom_frame=odom, base_link_frame=base_link, publish_tf=true. Inspect config/ekf.yaml boolean arrays and ensure message conventions/covariances are valid.

ros2 topic echo /odometry/filtered --once
ros2 topic hz /odometry/filtered
ros2 run tf2_ros tf2_echo odom base_link
ros2 run tf2_tools view_frames

Checks fused odometry, rate, odom-to-base transform and TF graph. EKF should be the only publisher of odom → base_link.

| Saved transform example | Translation | Rotation |
| --- | --- | --- |
| base_link → imu_link | (0,0,0.05) m | (0,0,0) |
| base_link → laser | (0.18,0,0.12) m | (0,0,0) |
| base_link → camera_link (legacy) | (0.1,0,0.2) m | (0,0,0) |

These are example values, not verified measurements. Replace legacy camera frame with actual D421 frame and measured mount pose.

# 11. ESP32 firmware and motor connections

Use esp32_driver/ in Arduino IDE for ESP32-WROOM-32. rpi_motor_driver/ is the legacy Pi Zero WH controller and is not used for direct motor control in the ESP32 build. Pan/tilt features are removed.

Install Arduino IDE and Espressif ESP32 board support using current Espressif instructions.

Select the correct ESP32 DevKit and USB port; compile/upload firmware.

Ensure Bluetooth command strings in bt_mobile_app-ESP32version/ match the firmware. Pi UART command framing and baud rate must also match.

Connect Pi TX → ESP32 RX, ESP32 TX → Pi RX, and GND ↔ GND. Use 3.3 V TTL.

Check exact OPEN-SMART DRV8833 labels and any sleep/enable pins against the actual board schematic.

Test one motor at a time with wheels lifted and low PWM.

| UART | Pi 5 | ESP32 proposal |
| --- | --- | --- |
| Pi TX → ESP RX | GPIO14 TX; physical pin 8 only if configured | GPIO16 RX2 |
| Pi RX ← ESP TX | GPIO15 RX; physical pin 10 only if configured | GPIO17 TX2 |
| Ground | GND | GND |

| Motor inputs | Proposed ESP32 GPIO |
| --- | --- |
| Left driver IN1/IN2/IN3/IN4 | 26 / 27 / 14 / 13 |
| Right driver IN1/IN2/IN3/IN4 | 25 / 33 / 32 / 23 |

These assignments are illustrative, not verified against the current sketch. Check UART configuration, baud rate, PWM support, boot-strapping pins and board pads. Never connect 5 V encoder outputs directly to Pi GPIO.

# 12. Power and electrical safety

Electronics LiPo (recorded proposal 7.4 V, 5200 mAh) feeds suitable converters for Pi 5 and ESP32/sensors. Pi 5 needs stable regulated 5 V with peak-current headroom.

Motor LiPo (recorded proposal 7.4 V, 5200 mAh) feeds driver VM only if allowed by the exact breakout/chip specification.

Never feed 7.4 V directly to Pi 5 5 V input, ESP32 5V/VIN or sensor logic power. Measure converter outputs before connecting devices.

Use common signal ground for Pi/ESP32 UART and driver logic as required. Do not accidentally connect battery positive rails together.

Use suitable fuses, polarity protection, rated connectors and strain relief. Separate encoder/I²C wiring from motor wiring.

Use a LiPo-compatible charger and never charge packs unattended.

Motor-start power dips were reported previously. Capacitors may help transients, but do not compensate for undersized converters, high-resistance wiring or inadequate battery/driver current ratings.

# 13. RealSense D421 setup

Recorded packages: ros-jazzy-realsense2-camera 4.58.4 and ros-jazzy-librealsense2 2.58.4. The D421 was detected; depth and left/right IR topics were available; RGB was unavailable. USB 2.1 connectivity was observed, reducing throughput.

sudo apt update
sudo apt install -y ros-jazzy-realsense2-camera ros-jazzy-librealsense2

Installs the recorded driver packages if available.

source /opt/ros/jazzy/setup.bash
ros2 launch realsense2_camera rs_launch.py

Starts camera defaults; inspect supported parameters before adding optional launch arguments.

ros2 topic list | grep camera
ros2 topic list | grep -E "infra1|infra2|depth|camera_info"
ros2 topic echo /camera/camera/infra1/camera_info --once
ros2 topic echo /camera/camera/infra2/camera_info --once

Previously observed: /camera/camera/depth/image_rect_raw, /camera/camera/infra1/image_rect_raw, /camera/camera/infra2/image_rect_raw and matching camera-info topics. Confirm live names.

Do not force enable_color or remap IR to RGB: the D421 has no conventional RGB stream. Use left/right rectified IR stereo input. Verify camera TF and avoid duplicate static transforms.

# 14. LiDAR SLAM with SLAM Toolbox

This is 2D LiDAR SLAM, not visual SLAM. It is a useful baseline before debugging camera mapping.

sudo apt install -y ros-jazzy-slam-toolbox
source /opt/ros/jazzy/setup.bash
ros2 launch sllidar_ros2 sllidar_c1_launch.py

Install SLAM Toolbox and start the LiDAR driver once, unless already launched by bringup.

source /opt/ros/jazzy/setup.bash
ros2 launch slam_toolbox online_async_launch.py use_odom:=false base_frame:=base_link map_frame:=map scan_topic:=/scan scan_qos:=sensor_data

Recorded troubleshooting command; launch arguments may vary by installed version.

ros2 topic info /scan
ros2 topic echo /scan --once
ros2 topic list | grep -E "^/map|slam_toolbox"
ros2 topic echo /map --once

Checks scan and map publication. In RViz set fixed frame to map when available; add Map, LaserScan, TF and RobotModel displays.

## SLAM Toolbox troubleshooting

Failed to Compute ODOM: check scan timestamp TF, odometry and configured base frame.

TF_OLD_DATA: inspect system clocks, simulated-time settings and stale/duplicate TF publishers; Chrony was considered during earlier troubleshooting.

Map appears but does not update: inspect scan rate, TF connectivity at scan timestamps, frame IDs and whether the robot is moving.

A fake odometry base frame was changed to base_footprint in a prior test and reported to help; do not retain fake odometry unless intentionally designed.

Avoid multiple mapping nodes competing to publish the same map or TF.

# 15. Stereo Visual SLAM with RTAB-Map and D421

The saved bringup is not a verified D421 visual-SLAM launch: it remapped RTAB-Map rgb/image to legacy /image_raw and included --delete_db_on_start. Replace the RGB input with stereo IR inputs before claiming visual SLAM works.

sudo apt update
sudo apt install -y ros-jazzy-rtabmap-ros

Installs RTAB-Map ROS integration if available; this package was previously installed.

## Step A — verify stereo topics

ros2 topic list | grep -E "infra1|infra2|camera_info"
ros2 topic info /camera/camera/infra1/image_rect_raw
ros2 topic info /camera/camera/infra2/image_rect_raw
ros2 topic echo /camera/camera/infra1/camera_info --once
ros2 topic echo /camera/camera/infra2/camera_info --once

Both rectified IR images and matching camera-info topics must publish. Check dimensions, timestamps, frame IDs and calibration. Use actual topic names from the running driver.

## Step B — calibration, synchronization and TF

Use rectified left/right IR images; do not label IR as RGB.

Check left/right image timestamps and rates for synchronization.

Verify stereo calibration and baseline from camera-info/driver output.

Connect camera optical frames to base_link using measured mounting transform; avoid duplicate transforms.

Choose stereo odometry and mapping parameters compatible with installed RTAB-Map; RGB-D defaults do not automatically apply.

## Step C — inspect installed launch support

ros2 pkg executables rtabmap_slam
ros2 pkg executables rtabmap_odom
ros2 pkg prefix rtabmap_launch
find /opt/ros/jazzy/share -maxdepth 3 -iname "*stereo*" 2>/dev/null | grep -i rtabmap

Lists executables and searches installed stereo examples. Package/launch names vary. Inspect actual installed launch arguments, e.g. ros2 launch <package> <launch_file> --show-args where supported, then configure left/right images, camera info, base frame and odometry.

## Step D — verify outputs

ros2 topic list | grep -E "rtabmap|map|odom|camera"
ros2 topic echo /odometry/filtered --once
ros2 run tf2_ros tf2_echo odom base_link
ros2 run tf2_tools view_frames

Acceptance: both IR streams and calibration are valid, synchronization works, camera TF is connected, odometry is continuous, RTAB-Map consumes stereo data without errors, and pose/graph/map changes as the robot moves.

Database warning: --delete_db_on_start removes previous RTAB-Map data at startup. Remove it for persistent sessions; keep only when intentionally starting fresh.

# 16. RViz2 on separate Ubuntu VM

Project used Ubuntu 24.04 in VirtualBox on Windows with a bridged adapter while sensors/SLAM ran on Pi 5. Both machines need compatible ROS domain and DDS/network settings.

source /opt/ros/jazzy/setup.bash
printenv | grep -E "ROS_DOMAIN_ID|RMW_IMPLEMENTATION|CYCLONEDDS_URI"
ros2 node list
ros2 topic list

Checks ROS environment and discovery. Match ROS_DOMAIN_ID and allow DDS traffic through VM bridge/firewall. If nodes appear but data do not, inspect multicast, network interface, RMW/DDS settings and clock consistency.

Cyclone DDS and XML configuration were used during earlier troubleshooting; warnings/deprecation notices were seen. Validate old XML against the installed Cyclone DDS version.

# 17. Daily diagnostic sequence

# Terminal 1: start verified bringup
source /opt/ros/jazzy/setup.bash
source ~/Desktop/bno085_ws/install/setup.bash
source ~/Desktop/odom/ros_ws/install/setup.bash
source ~/Desktop/slam/install/setup.bash
source ~/Desktop/robot_bringup_ws/install/setup.bash
ros2 launch robot_bringup bringup.launch.py

# Terminal 2: topic rates
source /opt/ros/jazzy/setup.bash
ros2 topic list
ros2 topic hz /scan
ros2 topic hz /imu/data
ros2 topic hz /odom_raw
ros2 topic hz /odometry/filtered

# Terminal 3: TF
source /opt/ros/jazzy/setup.bash
ros2 run tf2_tools view_frames
ros2 run tf2_ros tf2_echo odom base_link

# Terminal 4: RViz
source /opt/ros/jazzy/setup.bash
rviz2

Run each sensor driver only once. Launch stereo RTAB-Map only after camera inputs and TF are validated; do not run both SLAM Toolbox and RTAB-Map on the same TF/map without an intentional design.

# 18. Troubleshooting history

| Issue | Recorded lesson / checks |
| --- | --- |
| Map stopped updating | Previously reported resolved; recheck scan/TF timestamps, base frame, scan rate and duplicate publishers. |
| Failed to Compute ODOM | Check scan-time TF, odometry and base frame; base_footprint was used in a prior fake-odom test. |
| TF_OLD_DATA | Inspect clocks, simulated time and stale/duplicate TF publishers. |
| RPLidar missing scan | Check USB, permissions, driver process, /scan publisher and laser TF. |
| BNO085 not detected | Prior I²C scan found no address; check power, SDA/SCL, bus and breakout. |
| lgpio cannot open gpiochip | Check Pi 5 GPIO library, permissions, gpiochip and pin map. |
| UART garbage/overlong commands | Common Pi/ESP32 ground resolved earlier; also verify crossed TX/RX, baud and 3.3 V framing. |
| Motor-start power dips | Check converter peak current, battery, wiring resistance and driver rating. |
| RTAB-Map wants /image_raw | Legacy RGB configuration; D421 stereo requires left/right IR and camera info. |

# 19. Final safety and acceptance checklist

Measure buck-converter outputs before connecting electronics.

Verify battery polarity, fusing, connectors and DRV8833 voltage/current limits.

Confirm exact DRV8833 pad labels and motor output pairs from board documentation.

Test each motor direction at low PWM with wheels raised.

Verify encoder A/B signals and safe voltage levels for Pi GPIO.

Calibrate four-wheel mecanum odometry; confirm /odom_raw responds to movement.

Verify /scan, /imu/data, /odometry/filtered and TF tree.

Confirm D421 stereo images, camera info, calibration, timestamps and camera TF.

Remove legacy RGB remap; confirm RTAB-Map pose/graph changes as robot moves.

Disable database deletion if maps must persist.

Test stop command, communication timeout and emergency stop before autonomous operation.

# 20. Project folders and references

| Folder | Purpose |
| --- | --- |
| bt_mobile_app/ | Original Android Bluetooth driving app. |
| bt_mobile_app-ESP32version/ | App variant intended to match ESP32 firmware commands. |
| esp32_driver/ | Arduino firmware for ESP32 motor control. |
| ros2_pkgs/ | imu_node_pkg, wheel_odometry and robot_bringup. |
| rpi_motor_driver/ | Legacy Pi Zero WH motor-control code; not used for direct control in ESP32 build. |

Ubuntu Pi image: https://ubuntu.com/download/raspberry-pi

ROS 2 Jazzy installation: https://docs.ros.org/en/jazzy/Installation/Ubuntu-Install-Debs.html

RealSense ROS wrapper: https://github.com/IntelRealSense/realsense-ros

RTAB-Map ROS: https://github.com/introlab/rtabmap_ros

SLAM Toolbox: https://github.com/SteveMacenski/slam_toolbox

ESP32 Arduino core: https://docs.espressif.com/projects/arduino-esp32/en/latest/

Final note: this is a consolidated operational guide, not a manufacturer-verified netlist. Verify the ESP32 sketch GPIOs, actual DRV8833 board pad layout, encoder pin map, mecanum odometry and RTAB-Map stereo launch on the current hardware before autonomous use.